Yearly Maintenance Check

Kit (tool box top tray works well)

___	Oilcan

___	Grease

___	Cleaning rags

___	Allen wrenches

___	Cover removal tools (screwdrivers & wrenches)

___	Note pad and pencil (for noting big things & checking off items)

___	Small flashlight

___	Inspection mirror

___	Clipboard with this list attached

Table saw

___	Blow out

___	Oil trunnions & gears

___	Tighten setscrews on handwheels

___	Check for loose pulley setscrews & belt wear

___	Inspect plug, cord, & starter

Radial Arm Saw

___	Clean tracks & oil

___	Inspect plug, cord, & starter

Bandsaw

___	Check for loose pulley setscrews & belt wear

___	Blow out

___	Check guides for wear

___	Oil upper wheel carriage

___	Inspect plug, cord, & starter

Scroll saw

___	Check for loose pulley setscrews & belt wear

___	Blow out

___	Check oil level

___	Drop of oil in upper plunger

___	Inspect plug, cord, & starter

Bench grinders

___	Blow out

___	Check wheels

___	Inspect plug, cord, & starter

Lathe

___	Check for loose pulley setscrews & belt wear

___	Clean Morse taper sockets

___	Clean and oil tailstock clamp mechanism

___	Inspect plug, cord, & starter

Jointer

___	Check for loose pulley setscrews & belt wear

___	Clean and oil working parts on fence

___	Inspect knives

___	Inspect plug, cord, & starter

Planer

___	Check for loose pulley setscrews & belt wear

___	Check belts

___	Grease fittings, oil moving parts

___	Check knives

___	Inspect plug, cord, & starter

Shaper

___	Check for loose pulley setscrews & belt wear

___	Clean and oil working parts.

___	Inspect plug, cord, & starter

Drill press

___	Check for loose pulley setscrews & belt wear

___	Clean and oil working parts

___	Check chuck jaws

___	Inspect plug, cord, & starter

Mortiser

___	Inspect plug, cord, & starter

___	Oil working parts

Tenoner

___	Inspect plug, cord, & starter

___	Clean track

___	Oil working parts

Sander

___	Check for loose pulley setscrews & belt wear

___	Inspect plug, cord, & starter

___	Check oil level on OSS

___	Oil moving parts

Compressor

___	Check for loose pulley setscrews & belt wear

___	Check oil

___	Drain tank

___	Inspect plug, cord, & starter

Power Feeder

___	Check oil level

___	Inspect wheels, gears and chain

___	Inspect plug, cord, & starter

Oil working parts

Portable Power Tools

___	Gearbox oil (where needed)

___	Cords

___	Brushes